COURSE TITLE (i.e., EE809 Analog IC Design)

MSEE (ICS) 2026

Tutorial # 3

AC characteristics of MOS Transistors

| Name | Zuha Fatima |
| --- | --- |
| Reg # | 582671 |
| Instructor | Dr. Saman Fatima |
| Date | 26th September, 2026 |

Table of Contents

1. Introduction	2

2. Objectives	2

3. Guided tasks	2

3.1 Task A  Transconductance and current efficiency	2

3.1.1 Drain current and small signal parameters	3

3.1.2 Transition frequency from operating point data	3

3.1.3 Speed versus current efficiency	4

3.1.4 Intrinsic gain	5

3.1.5 Gain versus current efficiency	7

3.2 Task B  Transition frequency	7

3.2.1 AC verification	8

3.2.2 Channel length and width sweeps	8

3.2.3 Transition frequency versus overdrive	9

3.3 Task C  Maximum oscillation frequency	10

3.3.1 Effect of increasing device dimensions	11

3.3.2 Effect of two gate fingers	12

3.3.3 RF NMOS and finger count	13

4. Exercise activities	13

4.1 Activity 3.1  PMOS characterization	13

4.2 Activity 3.2  PMOS dimension sweeps	18

4.3 Activity 3.3  Speed efficiency FOM	18

4.4 Activity 3.4  PMOS maximum frequency	21

1. Introduction

Tutorial 3 examines MOS gain, speed and current efficiency through NMOS and PMOS simulations. The results show the trade-off between high frequency performance and efficient biasing.

2. Objectives

Extract gm/ID, intrinsic gain, fT and fmax; compare NMOS and PMOS behavior; study device dimensions and finger count; and identify the peak speed–efficiency FOM.

3. Guided tasks

3.1 Task A  Transconductance and current efficiency

Sweep VGS from 0 to 1 V with VDS = 0.5 V and grounded source and bulk. The supplied schematic shows W = 120 nm and L = 60 nm.

Figure 1. Guided Task A NMOS testbench with W = 120 nm and L = 60 nm.

Figure 2. Combined DC sweep of ID, gm, Cgg and gm/ID against VGS.

Separate plots clarify the traces because their units and scales differ.

3.1.1 Drain current and small signal parameters

Figure 3. Separate plots of ID, Cgg, gm and gm/ID against VGS.

At VGS = 1 V, ID ≈ 60 µA, gm ≈ 170 µS and Cgg ≈ 200 aF. Meanwhile, gm/ID falls from about 28 to 3 V⁻¹ as gate bias increases.

Figure 4. Isolated gm/ID curve showing the reduction in current efficiency as VGS increases.

3.1.2 Transition frequency from operating point data

The fT estimate rises with bias because gm increases faster than Cgg.

fT ≈ gm2πCgg

Figure 5. Calculated fT against VGS, reaching approximately 137 GHz at VGS = 1 V.

Figure 6. fT and gm/ID overlaid against VGS before constructing the wave-versus-wave plot.

The nearly flat gm/ID trace is caused by the shared scale, not constant efficiency.

3.1.3 Speed versus current efficiency

WaveVsWave plots fT against gm/ID at corresponding bias points.

Figure 7. fT versus gm/ID. Frequency decreases as transconductance efficiency increases.

Figure 8. Side-by-side comparison of fT versus gm/ID and fT versus VGS.

Higher gm/ID improves current efficiency but reduces fT. Moderate inversion provides a compromise.

3.1.4 Intrinsic gain

Intrinsic gain is the unloaded small signal gain magnitude:

Av0 = gmro

Figure 9. Intrinsic gain gmro against VGS, decreasing from approximately 14 to 4.4 V/V.

Figure 10. Initial wave-versus-wave result with intrinsic gain on the horizontal axis and gm/ID on the vertical axis.

Figure 10 has reversed axes: its horizontal quantity is gmro despite the VGS label. Figure 11 shows the requested orientation.

3.1.5 Gain versus current efficiency

Figure 11. Intrinsic gain against gm/ID, with gm/ID on the horizontal axis in V⁻¹.

Gain rises from about 4.4 to 14 V/V as gm/ID increases from 3 to 28 V⁻¹. Higher efficiency therefore gives more intrinsic gain but less speed.

3.2 Task B  Transition frequency

Compare the DC estimate and AC current-gain result near VGS = 0.6 V, with VDS = 0.5 V.

Figure 12. DC-derived fT marker at VGS = 601.14 mV: gm = 72.1791 µS, Cgg = 179.4788 aF and fT = 64.00757 GHz.

fT ≈ 72.1791 × 10⁻⁶2π × 179.4788 × 10⁻¹⁸ ≈ 64.0 GHz

Figure 13. AC testbench. V3 drives the gate, V1 sets VDS = 500 mV, and V0 ties VSS to ground; device dimensions use variables W and L.

The ideal drain bias source holds the drain at AC ground for short-circuit current-gain extraction.

3.2.1 AC verification

The AC transition frequency is the unity-current-gain point, or 0 dB:

Ai,dB = 20 log10|iDiG|

Figure 14. AC current-gain magnitude with the 0 dB crossing at 66.6 GHz.

The AC result is 66.6 GHz, about 4.1% above the 64.0 GHz DC estimate. Gain falls approximately 20 dB per decade.

The small difference reflects the DC approximation and AC terminal-current effects; the DC marker is also slightly offset at 601.14 mV.

3.2.2 Channel length and width sweeps

At VGS = 0.6 V, sweep L from 60 nm toward 1 µm, then W from 0.2 to 1 µm at L = 60 nm.

Figure 15. Calculated fT versus channel length. Increasing L sharply reduces transition frequency.

Increasing L reduces fT from about 64 GHz to roughly 1 GHz as channel transit time increases.

Figure 16. Calculated fT versus width at fixed length, spanning approximately 60.8 to 64.8 GHz.

Width causes only a modest variation, about 60.8–64.8 GHz, because gm and Cgg largely scale together.

3.2.3 Transition frequency versus overdrive

Overdrive is calculated from the gate and threshold voltages:

Vov = VGS - VTH

Figure 17. Transition frequency versus overdrive voltage. The frequency increases through turn-on and approaches a plateau at high overdrive.

The fT curve rises through turn-on and flattens at high overdrive, where additional bias gives little speed improvement.

The screenshot omits the frequency multiplier, so an absolute peak cannot be confirmed from its axis alone.

3.3 Task C  Maximum oscillation frequency

Use a two-port S-parameter sweep from 1 GHz to 2 THz at VGS = 0.6 V and VDD = 1 V. The near-zero Gmax crossing estimates fmax here.

Figure 18. Two-port nch testbench: W = 120 nm, L = 60 nm, one finger, external gate resistance of 50 Ω, 1 µF coupling capacitors and 1 µH bias inductors.

Coupling capacitors block DC; inductors feed bias while impeding RF current. The external gate resistor adds loss.

Figure 19. Minimum-size nch power-gain response. The marker is at 1.48958 THz and −18.17 mdB, close to the 0 dB crossing.

The initial crossing is approximately 1.49 THz and depends on the device model and parasitics.

3.3.1 Effect of increasing device dimensions

Increase W to 5 µm and L to 120 nm as specified in the tutorial.

Figure 20. Gmax versus frequency after increasing the device dimensions.

Figure 21. The same response with a marker at 323.593659 GHz and −1.275 mdB, essentially the unity-gain crossing.

The marked estimate falls to 323.6 GHz, about 78% below the initial value. Since both dimensions change, their individual effects are not isolated.

3.3.2 Effect of two gate fingers

Figure 22. nch response with two fingers, giving a marked unity-power-gain frequency of 235.0 GHz.

Two fingers reduce the crossing from 323.6 to 235.0 GHz, a 27.4% decrease.

More fingers can reduce gate resistance, but total width and capacitance may also change. The external 50 Ω resistor remains fixed.

The tutorial approximation relates these competing effects:

fmax ≈ fT8πRgCgd

Reported crossings use the displayed Gmax traces, not a separate unilateral-gain plot.

3.3.3 RF NMOS and finger count

The nmos_rf runs compare 16 and 30 fingers. The tutorial removes the external gate resistor because the RF model includes gate resistance.

Figure 23. nmos_rf with 16 fingers: the marked 0 dB crossing is 130.0 GHz.

Figure 24. nmos_rf with 30 fingers: the marked 0 dB crossing is 110.0 GHz.

Increasing the count from 16 to 30 lowers fmax from 130 to 110 GHz, a 15.4% decrease. More fingers do not guarantee greater speed.

4. Exercise activities

4.1 Activity 3.1  PMOS characterization

Repeat Task A using pch_mac with W = 200 nm, L = 60 nm and VSD = 0.5 V. Sweep VSG from 0 to 1 V.

Figure 25. PMOS bias testbench with source and bulk tied to VDD.

Figure 26. PMOS drain current versus VSG.

The drain-current magnitude reaches approximately 33 µA at VSG = 1 V. Its negative sign follows the terminal-current convention.

4.1.1 Transconductance and gate capacitance

Figure 27. Supplied gm trace versus the displayed VGS sweep.

Figure 28. PMOS gate capacitance versus the displayed gate-bias sweep.

Cgg falls to about 133 aF before rising to roughly 235 aF. The decreasing gm trace uses a VGS-labelled sweep and does not match the later increasing-VSG run; it should not be treated as the same bias sweep.

4.1.2 Current efficiency and inversion regions

Figure 29. Initial gm/ID trace under the displayed VGS sweep.

Figure 30. Labelled gm/ID versus VSG with approximate inversion regions.

The labelled VSG run shows gm/ID falling from about 28 to 3 V⁻¹, with moderate inversion around 0.5–0.65 V. The earlier VGS-labelled trace rises instead, indicating a different bias convention or run.

4.1.3 Speed and intrinsic gain

Figure 31. PMOS fT versus gm/ID.

Figure 32. PMOS intrinsic gain versus gm/ID.

As gm/ID increases, fT falls from about 76 GHz toward zero while intrinsic gain rises from roughly 4.2 to 13.3 V/V. Higher efficiency therefore trades speed for gain.

4.2 Activity 3.2  PMOS dimension sweeps

Figure 33. PMOS fT for L = 60–600 nm in 60 nm steps.

Increasing L reduces the displayed fT from about 215 GHz to roughly 6 GHz.

Figure 34. PMOS fT for W = 0.2–1 µm.

The width sweep dips near 0.3 µm, then rises to about 282 GHz at 1 µm. These runs have a different frequency range from Activity 3.1; matching bias and saved expressions is necessary for a direct comparison.

4.3 Activity 3.3  Speed efficiency FOM

Calculate FOM = fT × gm/|ID| while sweeping VGS from 0 to 1 V at VDS = 0.5 V.

Figure 35. NMOS FOM testbench showing W = 200 nm and L = 60 nm.

Figure 36. Saved gm, fT and signed source-current traces for the FOM calculation.

The supplied schematic uses W = 200 nm rather than the activity’s requested 120 nm. The source current is negative, so its magnitude is needed for a positive efficiency measure.

4.3.1 FOM peak

Figure 37. Signed FOM curve versus VGS.

Figure 38. FOM plot with a zero reference line.

The curve reaches approximately −720 GHz/V near VGS ≈ 0.67 V. Using current magnitude gives a peak FOM of about 720 GHz/V, showing the best speed–efficiency balance in this run. This is a visual estimate; the supplied marker does not identify the extremum.

4.4 Activity 3.4  PMOS maximum frequency

Figure 39. pch_mac two-port testbench with W = 200 nm, L = 60 nm and external Rg = 50 Ω.

Figure 40. pch_mac marker at 829.85 GHz and +45.95 mdB, near unity power gain.

The PMOS estimate is about 0.83 THz, below the initial NMOS value of 1.49 THz in Part C. Different device widths and models prevent attributing the difference solely to transistor polarity.

4.4.1 RF PMOS comparison

Figure 41. pmos_rf testbench with 16 fingers, 2 µm finger width and L = 60 nm.

Figure 42. RF PMOS marker at 38.90 GHz and −5.83 mdB, close to 0 dB.

The RF PMOS crossing is about 38.9 GHz versus 130 GHz for the 16-finger RF NMOS. However, this PMOS schematic retains the external 50 Ω gate resistor, unlike the Part C RF setup; the comparison includes that additional loss.